# RTL Optical Flow Accelerator: Performance, Area, and Power Characterization

## Taegon Hibbitts

University of Maryland — Computer Engineering

# 1. Introduction

This project explores the hardware acceleration of the structure tensor accumulation stage used in the Lucas–Kanade optical flow algorithm. The goal was to replace the computationally expensive software neighborhood accumulation stage with a streaming RTL accelerator capable of high throughput and efficient FPGA implementation.

The accelerator was implemented in Verilog, verified against a software reference using Verilator, synthesized in Vivado for FPGA characterization, and additionally synthesized using Yosys for ASIC-style logical area analysis.

The project focuses not only on correctness and speedup, but also on the architectural tradeoffs between throughput, area, buffering cost, and power consumption.

# 2. Background

Lucas–Kanade optical flow computes local motion vectors using image gradients and a structure tensor accumulation window.

For each pixel, the following quantities are accumulated over a neighborhood:

- Sxx = Σ(Ix²)

- Sxy = Σ(IxIy)

- Syy = Σ(Iy²)

- Sxt = Σ(IxIt)

- Syt = Σ(IyIt)

A software implementation performs repeated neighborhood summations over a sliding window, which becomes computationally expensive for large images.

The accelerator replaces these repeated software accumulations with a streaming box-filter architecture.

# 3. High-Level RTL Architecture

The accelerator implements a fully streaming 5×5 accumulation pipeline.

## Pipeline Stages

- Gradient product generation

- Horizontal 5-tap box filter

- Vertical line-buffered accumulation

- Streaming tensor output

The architecture processes one pixel per cycle after pipeline fill.

## Major RTL Modules

| Module | Purpose |
| --- | --- |
| tensor_accel | Top-level accumulation pipeline |
| box_filter_h5 | Horizontal running-sum filter |
| line_buffer_5 | Stores previous rows for vertical accumulation |
| box_filter_v5 | Vertical accumulation stage |

# 4. Design Decisions and Justification

## 4.1 Streaming Architecture

### Decision

A fully streaming architecture was selected instead of frame-buffered processing.

### Justification

Streaming architectures minimize repeated memory accesses and maximize throughput by allowing continuous pixel flow through the pipeline.

### Quantitative Result

- Sustained throughput: ~1 pixel/cycle

- Estimated throughput: ~122 MPixels/sec

### Implications

Pros:

- High sustained throughput

- Minimal control overhead

- Good scalability for larger image sizes

Cons:

- Requires line buffers and persistent image state

- Increases storage overhead

- Increases switching activity due to continuous operation

## 4.2 5×5 Window Size

### Decision

A 5×5 accumulation window was used.

### Justification

Larger accumulation windows improve robustness against image noise and improve optical flow stability.

### Quantitative Result

The design maintains timing closure at:

- 100 MHz target frequency

- ~122 MHz estimated Fmax

### Implications

Pros:

- Better smoothing and accumulation quality

- Increased numerical stability

Cons:

- Larger adder chains

- Increased line-buffer storage

- Increased critical path pressure

The accumulation chain became the dominant timing path in synthesis.

## 4.3 Fixed-Point Arithmetic

### Decision

The accelerator uses:

- signed 16-bit gradients

- signed 32-bit accumulators

### Justification

Fixed-point arithmetic substantially reduces hardware cost relative to floating-point implementations while preserving sufficient numerical precision for structure tensor accumulation.

### Quantitative Result

Resource usage remained low:

- ~515 LUTs

- ~459 registers

- 1 DSP block

### Implications

Pros:

- Lower area

- Lower power

- Higher achievable frequency

Cons:

- Reduced numerical precision relative to floating point

- Potential overflow risk if accumulation ranges grow significantly

## 4.4 Line-Buffered Window Generation

### Decision

The accelerator uses line buffers to maintain streaming vertical window state.

### Justification

A streaming architecture requires access to previous rows without repeatedly accessing external memory.

### FPGA Quantitative Result

Vivado mapped line buffers primarily into:

- SRLs

- distributed RAM

- minimal BRAM usage

### ASIC Quantitative Result

ASIC-style synthesis mapped line buffers into large numbers of synthesized storage elements.

Example:

- line_buffer_5 ASIC area: ~25,374

- box_filter_h5 ASIC area: ~675

- box_filter_v5 ASIC area: ~722

### Implications

Pros:

- Enables one-pixel-per-cycle throughput

- Efficient on FPGA fabrics due to BRAM/SRL support

Cons:

- Storage dominates logical area in ASIC-style synthesis

- Data movement and buffering become major hardware costs

This result demonstrates a key accelerator-design insight:

Data movement and buffering can dominate hardware cost more than arithmetic itself.

## 4.5 Pipelined Datapath

### Decision

The design uses a deeply pipelined datapath.

### Justification

Pipelining improves achievable frequency and sustained throughput.

### Quantitative Result

- ~459 registers used

- Timing closure achieved at 100 MHz

- Estimated Fmax ≈ 122 MHz

### Implications

Pros:

- High throughput

- Continuous streaming operation

- Reduced combinational critical path depth

Cons:

- Increased register switching activity

- Increased dynamic power consumption

## 4.6 Minimal DSP Utilization

### Decision

The design intentionally uses minimal DSP resources.

### Quantitative Result

- DSP48E1 usage: 1

### Implications

Pros:

- Preserves FPGA DSP resources

- Increases portability across FPGA families

- Simplifies synthesis portability

Cons:

- More arithmetic mapped into LUT/carry logic

- Potentially higher switching power

- Increased LUT utilization

The synthesis reports showed that carry-chain arithmetic became the dominant timing and logic component.

# 5. Functional Verification

## Methodology

The RTL accelerator was verified against a software reference implementation using Verilator.

The verification flow: 1. Streamed image data into RTL 2. Computed software reference output 3. Compared outputs cycle-by-cycle

## Result

- mismatches = 0

## Interpretation

The accelerator correctly reproduces the causal 5×5 software accumulation behavior.

# 6. CPU Baseline Performance

## Benchmark Platform

- 12th Gen Intel Core i5-1240P

- Optimized native C++ implementation

## Measured Throughput

| Resolution | Avg Runtime | Throughput |
| --- | --- | --- |
| 256×256 | 3.918 ms | 16.7 MP/s |
| 512×512 | 17.476 ms | 15.0 MP/s |
| 1024×1024 | 62.442 ms | 16.8 MP/s |

## Observation

CPU throughput remained relatively stable across tested image sizes.

# 7. FPGA Timing and Throughput

## FPGA Target

- Artix-7 xc7a35tcpg236-1

## Timing Constraint

- 100 MHz target clock

## Timing Results

- Worst Negative Slack (WNS): +1.840 ns

- Estimated critical path delay: ~8.16 ns

- Estimated Fmax: ~122 MHz

## Throughput

The architecture produces one output per cycle after pipeline fill.

Estimated throughput:

- ~122 MPixels/sec

# 8. Performance Comparison

## Kernel-Level Speedup

| Implementation | Throughput |
| --- | --- |
| CPU | ~16 MP/s |
| RTL Accelerator | ~122 MP/s |

Estimated kernel-level speedup:

- ~7.6×

## System-Level Speedup

Profiling indicated that structure tensor accumulation accounted for approximately 48% of total runtime.

Using Amdahl’s Law:

Estimated full-application speedup:

- ~1.7×

## Implications

The accelerator substantially improves the targeted kernel, but overall application acceleration is limited by remaining software stages.

# 9. FPGA Resource Utilization

## Vivado Synthesis Results

| Resource | Usage |
| --- | --- |
| LUTs | ~515 |
| Registers | ~459 |
| DSP48E1 | 1 |
| BRAM | 1 |
| SRL16E | 32 |
| CARRY4 | 47 |

## Interpretation

The accelerator is:

- arithmetic-heavy

- moderately pipelined

- memory-assisted

Carry-chain logic dominates arithmetic implementation.

Line buffering efficiently maps into FPGA memory-oriented structures.

# 10. Power Analysis

## Methodology

Power estimation used:

- routed FPGA implementation

- SAIF-driven activity annotation

- Vivado activity-based power analysis

Two operating modes were evaluated:

- idle

- active streaming

## Idle Power

| Metric | Value |
| --- | --- |
| Total | 72 mW |
| Dynamic | 3 mW |
| Static | 68 mW |

## Active Streaming Power

| Metric | Value |
| --- | --- |
| Total | 114 mW |
| Dynamic | 42 mW |
| Static | 72 mW |

## Dynamic Increase

Active operation increased dynamic power by approximately:

- ~39 mW

## Interpretation

The increase in dynamic power is primarily caused by:

- arithmetic switching

- line-buffer activity

- continuous pipeline operation

Static FPGA leakage dominates total power during idle operation.

# 11. Power Tradeoffs

## Observation

Power consumption scales significantly with switching activity.

The active streaming workload exercises:

- carry-chain arithmetic

- line-buffer updates

- register toggling

- routing activity

## Architectural Implication

The streaming architecture achieves high throughput, but sustained arithmetic and buffering activity increase dynamic power consumption.

## Hierarchical Power Insight

Power analysis showed that:

- line buffers contributed significantly to dynamic power

- arithmetic datapaths were major switching sources

This reinforces the architectural observation that buffering and data movement are major accelerator costs.

# 12. ASIC-Style Synthesis Analysis

## Methodology

The RTL was additionally synthesized using:

- Yosys

- Nangate45 standard-cell library

This analysis was intended to estimate logical complexity and architectural scaling behavior.

## Important Limitation

The synthesis flow does not use dedicated SRAM macros.

As a result:

- line buffers are synthesized into flip-flops

- reported area reflects logical complexity rather than optimized physical ASIC area

## ASIC-Style Area Results

| Module | Area |
| --- | --- |
| box_filter_h5 | ~675 |
| box_filter_v5 | ~722 |
| line_buffer_5 | ~25,374 |
| tensor_accel total | ~142,841 |

## Interpretation

The computational filters themselves are relatively compact.

The dominant hardware cost comes from maintaining streaming image state through line buffering.

## Architectural Insight

This synthesis flow highlights a major distinction between FPGA and ASIC implementations:

| FPGA | ASIC-style synthesis |
| --- | --- |
| BRAM/SRL efficient | storage dominates |
| memory abstracted by fabric | memory explicit in logic |
| low apparent storage cost | large synthesized storage cost |

# 13. Overall Architectural Tradeoffs

| Design Choice | Benefit | Cost |
| --- | --- | --- |
| Streaming pipeline | High throughput | Increased buffering |
| 5×5 window | Better smoothing | Larger adder chains |
| Fixed-point arithmetic | Lower area/power | Reduced precision |
| Deep pipelining | Higher Fmax | More switching activity |
| Minimal DSP usage | Preserves DSP resources | More LUT/carry logic |
| Line buffering | 1 px/cycle throughput | Storage-heavy architecture |

# 14. Conclusions

This project successfully demonstrated a streaming RTL accelerator for structure tensor accumulation in the Lucas–Kanade optical flow pipeline.

The accelerator achieved:

- functional correctness

- timing closure

- substantial kernel-level speedup

- low FPGA resource utilization

- activity-aware power characterization

- ASIC-style logical area analysis

Key quantitative results include:

| Metric | Result |
| --- | --- |
| Throughput | ~122 MPixels/sec |
| Kernel speedup | ~7.6× |
| Estimated system speedup | ~1.7× |
| Dynamic power increase | ~39 mW |
| LUT usage | ~515 |
| Registers | ~459 |
| DSP usage | 1 |

The project also demonstrated an important architectural principle:

The cost of data movement and buffering can dominate arithmetic complexity in streaming accelerator architectures.

Overall, the design trades increased storage and switching activity for high sustained throughput through a fully streaming one-pixel-per-cycle architecture.

# 15. Future Work

Potential future extensions include:

- DSP-oriented multiplier mapping

- randomized high-switching power characterization

- multi-channel tensor parallelism

- larger accumulation windows

- full optical-flow pipeline acceleration

- SRAM-aware ASIC synthesis flows

- energy-per-frame analysis

- deeper pipeline optimization